PHYSICS-GUIDED RESIDUAL LEARNING WITH

RECURRENT NEURAL NETWORKS FOR

ENHANCED AIRCRAFT STATE ESTIMATION

UNDER ATMOSPHERIC GUST DISTURBANCES

| Author One Organization Street Address City, Province, Postal author.one@email.com | Author Two Organization Street Address City, Province, Postal author.two@email.com | Author Three Organization Street Address City, Province, Postal author.three@email.com |
| --- | --- | --- |

Copyright © 2026 by the author(s). Permission granted to INCOSE to publish and use.

# Abstract

This paper presents a hybrid physics- and data-driven framework for state estimation of an aircraft longitudinal flight model under unknown atmospheric gust disturbances. The architecture combines a discrete-time Kalman Filter (KF), augmented with random-walk gust states and online adaptive measurement-noise covariance, with a two-layer Gated Recurrent Unit (GRU) residual observer. The KF supplies physically interpretable state estimates and uncertainty propagation, while the GRU learns the structured residual dynamics that the linear aircraft model cannot capture. On a five-state longitudinal model (forward velocity, vertical velocity, pitch rate, pitch angle, altitude) discretized at 100 Hz, the hybrid estimator reduces RMSE relative to a standalone KF by 95.0% in forward velocity, 85.7% in vertical velocity, 66.6% in pitch rate, 90.5% in pitch angle, and 95.0% in altitude. The GRU (24 hidden units total across two layers, under 2,000 trainable parameters) trains on a single 20-second trajectory containing a structured gust window and adds its correction directly to the KF’s one-step-ahead prediction, so the filter degrades gracefully to classical Kalman behaviour whenever the network output is zero. Results show the residual-learning approach preserves KF robustness, interpretability, and uncertainty propagation while delivering substantial accuracy gains under unmodeled disturbances, at negligible computational cost.

# Keywords

Kalman filter, recurrent neural networks, gated recurrent units, aircraft state estimation, atmospheric disturbances, hybrid estimation

# Introduction

Every modern commercial aircraft relies on a recursive state estimator to feed reliable, continuous state information to autopilot systems, structural health monitors, and ground based situational-awareness tools. The linearized longitudinal dynamics of a large transport aircraft at cruise are well characterized by first-principles aerodynamic models, and under nominal conditions a properly tuned Kalman filter (KF) delivers near-optimal estimates at modest computational cost – the industry standard for avionics and flight management.

The trouble begins when the aircraft encounters an atmospheric gust. Gusts introduce structured, time-correlated effects that violate the white-noise process-noise assumption underlying classical Kalman filtering. When a persistent disturbance is active, the filter’s innovation sequence becomes biased; the filter responds by inflating its gain, which amplifies measurement noise across every state. States that are not directly measured – altitude in particular – suffer most, since biased intermediate estimates accumulate through kinematic integration.

The disappearance of Malaysia Airlines MH370 in March

2014 illustrated this problem starkly: over oceanic airspace with no continuous radar coverage, ground-station-in-theloop architectures must reconstruct the trajectory from sparse, bandwidth-limited measurement streams. Superimposing atmospheric disturbances on top of this sparsity compounds estimation error and degrades situational awareness and control authority.

The standard physics-based remedy is state augmentation: append estimated gust components to the state vector and model them as random walks. This works well when gusts vary slowly relative to the estimation bandwidth but struggles with time-varying amplitude or spectrally rich turbulence. A more recent family of hybrid methods instead replaces internal KF components – the gain matrix or the state-transition model with learned neural surrogates [1,2]. These approaches improve accuracy but sacrifice the uncertainty quantification and physical interpretability that make Kalman filters suitable for certification in safety critical aviation systems.

This work takes a deliberately conservative design stance. The Kalman filter is left completely intact – its gain computation, state transition, and covariance update follow the standard predict–update cycle unchanged. A small two layer GRU is inserted immediately after the filter’s one step-ahead prediction to add a residual correction. The GRU observes only the KF’s state predictions and the derivatives of the control inputs, then outputs an additive five-element correction that compensates for the structural model error the linear filter cannot represent. Because the learning target is a residual rather than the full state, the network generalizes from a short training sequence and is stable by construction: a zero-output GRU recovers the classical Kalman filter exactly.

The paper’s contributions are threefold: (1) an additive residual-observer architecture, with an explanation of why anchoring the correction to the KF prediction (rather than the posterior) is essential for numerical stability; (2) an online adaptive measurement-noise estimator that prevents filter drift during gust-free cruise while remaining responsive to disturbance transients; and (3) a quantitative evaluation on a high-fidelity longitudinal aircraft model showing that a network of under 2,000 parameters cuts estimation error by roughly two-thirds to nineteen-twentieths across all five states.

# Related Work

## Physics-Based Gust and Disturbance Estimation

State augmentation is the canonical approach for integrating wind and turbulence into aircraft state estimation. Early work by Stengel [3] and Nelson [4] appended wind states modeled as random walks to the aircraft state vector. These methods work well for slowly time-varying gusts but struggle with rapid amplitude changes or spectral complexity. Shaping filters parameterized by Dryden or von Karm´ an´ spectra [5] give frequency-domain correspondence to real turbulence but require prior knowledge of the disturbance spectrum, and the Interacting Multiple Model (IMM) filter [6] improves robustness at the cost of enumerating all plausible disturbance regimes in advance.

Adaptive Kalman filtering tunes process or measurement noise covariance online from innovation statistics [7], but simultaneous identification of both Q and R is ill conditioned without added constraints. This work sidesteps that difficulty by fixing process noise (grounded in physics) and adapting only measurement noise online.

## Recurrent Neural Networks in Aerospace Estimation

LSTM and GRU architectures [8, 9] are increasingly used in aeronautical estimation, from inertial navigation to GPS denied flight, because their internal memory suppresses long-term drift and models temporal correlation in sensor data. GRUs are attractive for resource-constrained avionics: fewer gates and parameters than an LSTM for comparable expressiveness. The persistent limitation of purely data-driven estimators is the absence of uncertainty quantification, which complicates certification for flight-critical use.

## Hybrid Kalman Filter Architectures

Recent hybrid architectures merge classical filtering with learned components. KalmanNet [1] replaces the Kalman gain with a learned network, improving responsiveness to non-stationary noise but obscuring the gain matrix’s uncertainty semantics. Deep Kalman Filters [2] replace the linear state-transition model with learned latent dynamics, trading interpretability for expressivity. Bayesian neural-Kalman approaches [10] combine probabilistic neural models with filtering to recover some uncertainty estimates under degraded sensing.

The present framework differs from all three: it leaves the entire KF structure untouched and adds a GRU-based residual correction strictly after prediction, so uncertainty propagation and physical interpretability are preserved rather than traded away.

# Aircraft Model and Gust Disturbance

## Longitudinal Dynamics

The aircraft longitudinal dynamics [MIT paper] are a standard small perturbation linearization about a cruise trim point (forward velocity U0 = 235.9 m/s, air density ρ = 0.3045 kg/m3, pitch angle θ0 = 0). The five-state vector is forward velocity perturbation u (m/s), vertical velocity w (m/s), pitch rate q (rad/s), pitch angle θ (rad), and altitude h (m):

x=Acx+Bcu+Bwωg	(1)

where u=δe,δpTis the control vector (elevator deflection, normalized thrust) and wg=ug,wg,qgT is the unknown gust vector entering through gust-influence matrix Bw. The altitude state closes the system through the small angle kinematic relation h=-w+U0 θ.In expanded form (2),

| u.w.q.θ.h.=XuXw0-gcosγ0ZuZwU0-gsinγ0MuMwMq0000100-sinγcosγ0U0cosγ0uwqθh+XδeXδpZδeZδpMδeMδp00δeδp + -Xu-Xw0-Zu-Zw0-Mu-Mw-Mq000000ugwgqg |  |  |  |  |
| --- | --- | --- | --- | --- |

## Gust Disturbance Model

The gust is an unknown three-component disturbance injected between t = 10 s and t = 15 s: a 1 Hz headwind sinusoid (±10 m/s), a 1 Hz vertical-gust cosine (±5 m/s), and a constant pitch-rate upset (1.5 rad/s). Outside this window the gust is exactly zero. No gust measurement or feedforward signal is available to the estimator – all disturbance information must be inferred purely from noisy sensor innovations, matching realistic operations where the ground station cannot anticipate a gust.

## Discretization

The continuous-time dynamics are discretized at 100 Hz via zero-order hold, xk+1 = Adxk + Bduk. At this 10 ms interval the short-period mode (natural frequency ∼0.9 rad/s) is resolved with negligible phase lag, so the discrete model tracks gust transients accurately.

# Kalman Filter with Online Noise Adaptation

## State Augmentation

Since the estimator cannot directly observe the gust vector, three additional states representing the gust components [ug,wg,qg] are appended and modelled as random walks. The resulting augmented state vector contains the five aircraft states and three gust states. The augmented state-transition matrix is block-diagonal, with the discrete aircraft model (Ad) and a (3×3) identity matrix for the gust dynamics, while the measurement matrix observes only the flight states. The process-noise covariance (Q=diag(Qkf,Qg)) is also block-diagonal, using small values for the aircraft states to reflect confidence in the system model and unit values for the gust states to enable rapid tracking of the imposed 1 Hz gust disturbances.

Figure 1: Block diagram of the software Kalman filter loop

## Predict–Update Cycle

The filter runs the standard cycle every 10 ms:

Prediction:

| xk¦k-1=Adxk-1+Bduk | (5) |
| --- | --- |
| Pk¦k-1=AdPk-1AdT + Q Innovation and gain: | (6) |
| Yk=zk-Cxk¦k-1 | (7) |
| Sk=CPk¦k-1CT+Rk | (8) |
| Kk=Pk¦k-1CTSk-1 | (9) |

Update:

xk=xk|k-1+Kkyk	 	              (10)

Pk=I-KkCPk|k-1I-KkCT+KkRkKkT         (11)

## Online Adaptive Measurement Noise

To improve robustness under varying flight conditions, the measurement-noise covariance (R) is updated online using a 50-sample rolling innovation window. When the empirical innovation covariance exceeds the predicted covariance, (R) is smoothly inflated toward the measured value using a first-order exponential update with a forgetting factor of 0.95. As the filter re-converges, (R) gradually returns to its nominal value, while a minimum diagonal bound of (10-8) ensures numerical stability. This adaptive strategy maintains consistent filter performance across both steady-flight and turbulent conditions without manual re-tuning.

# GRU Residual Observer

## Motivation

The Kalman filter assumes memoryless Gaussian process and measurement noise. Atmospheric gusts violate this: their temporal correlation biases the innovation sequence persistently and causes the KF to drift. Rather than modify KF internals – which would sacrifice interpretability and covariance semantics – a small GRU is inserted after the prediction step to learn and correct exactly the structured error the linear model cannot represent. Targeting only the residual, rather than the full state, has three benefits: the network generalizes from a short training sequence, since residuals are structured and low-dimensional rather than arbitrary in magnitude; the system is stable by construction, since a zero-output GRU recovers the classical KF exactly; and the KF’s covariance semantics and uncertainty quantification are preserved through the update cycle.

## Network Architecture

At each step k the GRU input is

vk=xkKF;uk ϵ R7		         (14)

the five KF state predictions concatenated with the two control-input time derivatives. Including uk gives the network access to the rate of change of elevator and thrust commands, which correlate strongly with the aerodynamic transients that accompany gust onset and recovery.

Figure 2: Internal cell architecture of the Gated Recurrent Unit (GRU) correction layer

The network stacks two GRU layers, nh1 = 8 hidden units in layer 1 and nh2 = 16 in layer 2, followed by a linear output layer producing five state corrections (under 2,000 trainable parameters total). Layer 1’s forward pass at step k

is

| r1=σWr1vk+Ur1h1(k-1)+br1 (15) |  |
| --- | --- |
| z1=σWz1vk+Uz1h1(k-1)+bz1 (16) |  |
| h1=tanh(Wh1vk+Uh1r1⊙h1(k-1)+bh1) (17) |  |
| h1(k)=1-z1⊙h1k-1+z1⊙h1 (18) |  |

where σ· is the sigmoid, ⊙ the Hadamard product, and vk the normalized input to that layer. Layer 2 takes layer 1’s hidden state as input. The output layer produces the raw residual yk=Wyh2(k)+by ϵ R5.

Figure 3: Training block architecture, showing the stacked two-layer GRU trained offline via backpropagation through time, with the Kalman filter gain and state prediction supplied as inputs

Figure 4: Learning block architecture, showing the trained GRU integrated with the Kalman filter gain during inference to generate the final corrected state estimate.

A single layer is expressive enough for simple periodic patterns but struggles with the onset/decay transients at gust boundaries, where the reset gate must simultaneously unlearn the pre-gust periodic state and acquire the transient dynamics. The second layer captures the multi-step correlation between pitch dynamics and altitude accumulation that a single layer misses – and is the difference between a small accuracy gain and a large one (Section 7).

## Normalization and Target Scaling

Inputs are min-max normalized to -1,+1 using statistics from the training trajectory, with range components below 10-8 clamped to avoid division by zero. The training target at step k is the one-step-ahead residual, normalized per state:

yk*=xk+1true - (AdxkKF +Bduk)σtgt                      (19)

The scales reflect the expected magnitude of the gust induced residual in each state – large for velocities, small for angular states, large for altitude. Without this per-state scaling, the numerically larger altitude residual would dominate the loss and the network would neglect the subtler pitch corrections.

## Training

The network is trained by minimizing the mean squared error (MSE) over the complete (N)-step trajectory,

L=1N-2k=2N-1  yk - yk*2                       (20)

## Gradients are computed using full backpropagation through time (BPTT) without truncation. For the second GRU layer, the gate gradients at time step (k) are

δz2=∂L∂h2⊙h2-h2(k-1)⊙σ'(z2)              (21)

δh2=∂L∂h2⊙z2⊙tanh'(h2)               (22)

δr2=(Uh2Tδh2)⊙h2(k-1)⊙σ'(r2)               (23)

while the gradient propagated to the first layer is ∂L∂h1(k)=Wz2Tδz2+Wh2Tδz2+Wr2Tδr2.The first-layer gate gradients are computed analogously using the normalized input vk. All weights are initialized using Xavier uniform initialization [11] and updated using vanilla stochastic gradient descent (SGD) with a fixed learning rate of η=104). Momentum and adaptive optimizers are intentionally omitted because the compact network and normalized residual targets provide stable convergence while reducing the risk of overfitting to trajectory-specific noise.

## Inference and State Correction

At inference the corrected estimate is

xk+1GRU=AdxkKF+Bduk+yk⊙σtgt                    (24)

applied to the KF prediction, never the posterior – adding a correction after the measurement update would double-count sensor information, since the filter has already incorporated it. Grounding the correction to the prediction is equivalent to a pseudo-measurement with negligible noise on the model-error channel.

An alternative that propagates the corrected state recursively through Ad was tested and found numerically unstable: any error at step k is amplified by Ad at every later step and eventually diverges. Anchoring each correction to the fresh KF prediction breaks this accumulation chain and is essential for long-sequence stability.

# Experimental Setup

The simulation runs for T = 20 s at 100 Hz (N = 2,000 steps), integrated with MATLAB. Initial conditions represent a small perturbation from cruise trim, x0=5,10,0.01,0.08,200T . Control inputs follow a random walk, uk=uk-1+Tsuk, where uk∼0.01Ν(0,I2) representing mild autopilot excitation. Sensors observe [u,w,q,θ] with additive Gaussian noise (σ = 0.02 per channel); altitude is never measured directly and must be propagated through hk+1=hk+Ts(-wk+U0θk) which makes it the most challenging state for any estimator, since sustained bias in ω or θ during the gust integrates forward.

Three estimators are compared: (1) the baseline augmented KF with online adaptive R; (2) KF + single-layer GRU (8 hidden units, 1,085 parameters); (3) KF + stacked two-layer GRU (8+16 hidden units, the proposed method).

## Results

## State Estimation Accuracy

Figure 5: Comparison of state-estimation RMSE for the baseline KF, KF + GRU, and KF + stacked GRU estimators.

The stacked GRU improves every state, with the largest gains in forward velocity and altitude (95.0%) and the most conservative gain in pitch rate (66.5%). Altitude improves indirectly: it is never measured, so the accuracy gain comes from correcting the upstream w and θ biases that would otherwise integrate into altitude drift. The single-layer GRU is a useful negative result – it degrades altitude tracking rather than helping, because a shallow layer cannot decouple the cross-axis aerodynamic interaction during gust onset; its erroneous corrections to u propagate through kinematic integration into large altitude error. The second layer is what fixes this.

Figure 6: Training loss drops from ≈0.8 at initialization to under 0.01 within 30 epochs and settles near 0.003 by epoch 100 – fast convergence because the network learns only what the physics model gets wrong, not the full state dynamics from scratch.

Figure 7: Compares all three estimators against ground truth across the full run. Outside the gust window all three track closely; once the gust begins at t = 10 s, the baseline KF diverges, the single-layer GRU partially compensates, and the stacked GRU tracks truth closely through the disturbance and recovers smoothly afterward.

Figure 8: Isolates the error magnitude directly: outside the gust both estimators stay within a few m/s and a few hundredths of a radian, but inside the gust window the KF’s error grows sharply while the hybrid stays tightly bounded – the correction activates precisely when the disturbance is present and is negligible otherwise.

Figure 9: Zooming into the transient shows all estimators lag briefly during the rapid ramp-up (10–11 s); the KF then fails to recover and accumulates bias for the rest of the window, while both GRU variants respond with little lag, and the stacked GRU tracks the peak most tightly.

Figure 10: The adaptive-R mechanism stays near its nominal floor during gust-free cruise, inflates as soon as the gust raises the empirical innovation covariance, and relaxes back to nominal after the disturbance ends – confirming the filter re-tunes itself without manual intervention or divergence.

Figure 11: Innovation whitening is the diagnostic that ties the results together: a well-calibrated filter should produce white innovations (autocorrelation near zero except at lag 0). The raw KF innovations retain visible structure during the gust; once the GRU correction is applied, the residual autocorrelation collapses toward zero, confirming the network is removing the persistent bias rather than just averaging over it.

# Conclusion

This paper presents a hybrid estimator that combines a physics-grounded, fully intact Kalman filter with a lightweight two-layer GRU residual observer for aircraft state estimation under atmospheric gust disturbances. On a five-state longitudinal model subjected to a structured 5-second gust, the hybrid reduces RMSE by 66.5–95.0% across all states relative to the standalone KF, using under 2,000 trainable parameters that converge within 30 training epochs. The single-layer ablation is the clearest evidence that depth matters here: a shallow network not only fails to help altitude estimation but actively degrades it, because it cannot decouple the cross-axis aerodynamic dynamics that the second layer captures.

The broader point is that Kalman filters and recurrent networks are complementary rather than competing tools. The KF is the statistically optimal handler of stochastic Gaussian uncertainty, giving a certifiable, interpretable posterior covariance; the GRU is a parameter-efficient handler of structured, temporally correlated model error that the KF’s noise model cannot represent by design – closer to feedforward compensation in classical control, a learned correction for steady-state error that pure feedback cannot eliminate, than to a black-box replacement for the filter. Because the filter itself is left untouched, the covariance architecture – and with it the fault-detection and failure-mode-analysis pathways relevant to DO-178C/DO-254 compliance – is preserved; the GRU acts as a learned disturbance observer analogous to an augmented-state filter, not as an opaque decision layer. At under 2,000 parameters and roughly 5k FLOPS per sample, the network is cheap enough for resource-constrained avionics, and training on 20 seconds of data is compatible with existing certification test campaigns. This also means the approach can be layered onto any existing certified Kalman-based estimator without redesigning the underlying system – a practical advantage where legacy estimator certification represents a significant investment.

Two limitations bound how far these results can be read, and both point directly at what should come next. The evaluation here is transductive – the GRU is trained and tested on the same 20-second trajectory, which is physically reasonable for post-flight ground-station reconstruction but overstates generalization to unseen gust conditions – so the first priority is training across a library of gust profiles spanning different altitudes, weights, and atmospheric conditions, evaluated on held-out conditions and ideally against richer Dryden/von Karm´ an turbulence spectra rather than´ the structured sinusoid used here. Second, the network currently outputs only a point correction with no associated uncertainty, so an operator cannot tell whether the GRU is interpolating confidently or extrapolating past its training distribution; extending the output layer to predict a residual covariance and feeding it back into the KF update as a statistically consistent pseudo-measurement is the highestpriority architectural extension. Beyond these two, the natural next steps are to extend the framework to nonlinear flight regimes via an Extended or Unscented KF paired with a retrained GRU, to handle asynchronous or delayed measurements common in satellite-linked navigation, to validate against real flight-test data where unmodeled nonlinearities and sensor dynamics will stress the architecture in ways simulation cannot, and to develop rolling-window or real-time recurrent learning so the GRU keeps adapting online as the aircraft transitions through different flight regimes.

Taken together, these results support a simple thesis: for estimation problems where the dominant error is a structured, temporally correlated disturbance rather than sensor noise or an unmodeled nonlinearity, a small residual network layered on an otherwise-unmodified Kalman filter can recover most of the accuracy of a much more complex estimator while preserving the interpretability, certifiability, and computational footprint that flight-critical systems require.

# References

- G. Revach, N. Shlezinger, X. Ni, A. L. Escoriza, R. J. G. van Sloun, and Y. C. Eldar, "KalmanNet: Neural network aided Kalman filtering for linear systems," IEEE Signal Processing Letters, vol. 29, pp. 1341–1345, 2022.Nelson, R. C. (1998). Flight Stability and Automatic Control (2nd ed.). McGraw-Hill.

- R. G. Krishnan, U. Shalit, and D. Sontag, "Deep Kalman filters," in Proc. NIPS Workshop on Advances in Approximate Bayesian Inference, Montreal, Canada, Dec. 2015.Bar-Shalom, Y., Li, X.-R., & Kirubarajan, T. (1998). Estimation with Applications to Tracking and Navigation. John Wiley & Sons.

- R. J. Stengel, Flight Dynamics. Princeton University Press, Princeton, NJ, 2004.Hochreiter, S., & Schmidhuber, J. (1997). “Long Short-Term Memory.” Neural Computation, 9(8), 1735–1780.

- R. C. Nelson, Flight Stability and Automatic Control, 2nd ed. McGraw-Hill, New York, 1998.

- U.S. Department of Defense, MIL-HDBK-1797: Flying Qualities of Piloted Aircraft, Dec. 1997.

- Y. Bar-Shalom, X. R. Li, and T. Kirubarajan, Estimation with Applications to Tracking and Navigation. Wiley, Hoboken, NJ, 2001.

- M. S. Grewal and A. P. Andrews, Kalman Filtering: Theory and Practice with MATLAB, 4th ed. Wiley, Hoboken, NJ, 2015.

- S. Hochreiter and J. Schmidhuber, "Long short-term memory," Neural Computation, vol. 9, no. 8, pp. 1735–1780, Nov. 1997.

- K. Cho, B. van Merrienboer, D. Bahdanau, and Y. Bengio, "Learning phrase representations using RNN encoder–decoder for statistical machine translation," in Proc. EMNLP, Doha, Qatar, Oct. 2014, pp. 1724–1734.

- A. Gupta et al., "Neural aided Kalman filtering for UAV state estimation in degraded sensing environments," arXiv:2604.28107, Apr. 2026.

- X. Glorot and Y. Bengio, "Understanding the difficulty of training deep feedforward neural networks," in Proc. AISTATS, Sardinia, Italy, May 2010, pp. 249–256.

- S. Feng, X. Li, S. Zhang, Z. Jian, H. Duan, and Z. Wang, "A review: State estimation based on hybrid models of Kalman filter and neural network," Systems Science & Control Engineering, vol. 11, p. 2173682, 2023

- R. E. Kalman, "A new approach to linear filtering and prediction problems," Transactions of the ASME – Journal of Basic Engineering, vol. 82, pp. 35–45, 1960.

- J. P. How, Aircraft Longitudinal Dynamics, MIT OpenCourseWare, Department of Aeronautics and Astronautics, Massachusetts Institute of Technology, Cambridge, MA, USA, Course 16.333 Aircraft Stability and Control, Lecture Notes, 2004